author’s name: EME coordination and Compliance

business unit: Telstra operations

sub business unit:Network and access Technologies

Issue date: August 2012 ¦ issue number: 1

DOCUMENT NO: 005486

Procedure Document No: 005486
Telstra Site EME Compliance and Safe Work Procedure

Chapter 2 Attachment A04
RF Signage Application Note

SUMMARY: RF signage application note details correct RF signage placement for given scenarios

VERSION LABEL:

SECURITY CLASSIFICATION:

TABLE OF CONTENTS

1.	INTERNAL SIGNAGE: NON SITE SPECIFIC	3

2.	EXTERNAL SIGNAGE: TELSTRA & 3GIS – Poles, Towers and Masts	4

3.	EXTERNAL SIGNAGE: TELSTRA & 3GIS – Rooftops (EXCHANGES)	6

4.	EXTERNAL SIGNAGE: TELSTRA & 3GIS – MICRO CELL SITES	8

5.	EXTERNAL SIGNAGE: CUSTOMER END SITES – Masts and Poles and Roofmounts	9

6.	EXTERNAL SIGNAGE: CUSTOMER END SITES – Roof and Gutter and other mounts	10

7.	EXTERNAL SIGNAGE: NON TELSTRA STRUCTURE SITES	11

7.1.	Water Towers, poles and others	11

7.2.	Rooftops and accessible structures	12

7.3.	RF sources IBC’s and external antennas	14

7.4.	High Visual Amenity Sites	15

8.	Signage requirements of specific site owners:	16

# INTERNAL SIGNAGE: NON SITE SPECIFIC

| Description | Sign No | Sign |
| --- | --- | --- |
| Install tag to each of the switches or controls altered, feeders broken, or where a key interlock is provided, lock the switch and remove the key. | Telstra sign # 1 No MERCS equivalent Material #: Polyprop 148/00253 |  |

# EXTERNAL SIGNAGE: TELSTRA & 3GIS – Poles, Towers and Masts

| Telstra Site POLES |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
|  | Telstra Sign # 13 MERCS sign # 2 Material #: Polyprop 250 x 180 – 148/00331 JABEC ref TERH2331-P Metal 250 x 180 – 148/00330 JABEC ref TEMCF2330-M |  |
| This sign can be installed 1.5m above AGL to 3m below the lowest antenna mount on the pole of all Telstra & 3GIS owned & controlled sites. (eg. Greenfield sites, slimline pole structures). Note: Consideration should be given to not unduly alarming the passer by and the sign should be installed minimum 5 M above AGL for poles in publicly accessible areas such as sporting fields Fat Poles – Install sign on outside of access door. | Telstra sign # 6 No MERCS equivalent Material #: Adhesive PVC 400×350 - 141/00626 200×175 - 141/00627 METAL 400×350 - 148/00319 200×175 - 148/00320 |  |

NOTES:

1). 	RF EME warning Signs should not be installed on gates or external fences.

2). 	Note other than where mentioned all existing signage remain current until replacement is required" ie specifically, "DANGER RF Hazard Area" signs are "NOT VALID" and are "TO BE REPLACED".

3).	Telstra Sign No 8 & 9 are to be installed on High Powered sites.  eg. where Broadcast antennas are in-situ.

4).	Refer to MCF Document “7.1 Preferred Site Signage” on RFNSA “MERCS” document area for additional detail.

| Telstra site TOWERS & MASTS |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
|  | Telstra sign # 13 MERCS sign # 2 Material #: Polyprop 250 x 180 – 148/00331 JABEC ref TERH2331-P Metal 250 x 180 – 148/00330 JABEC ref TEMCF2330-M |  |
| Install this sign at the base of the structure on all Telstra & 3GIS owned & controlled sites. (eg. on ladder guard or tower support/leg) | Telstra Radhaz sign # 6 (No MERCS equivalent) Material #: PVC 400×350 - 141/00626 200×175 - 141/00627 METAL 400×350 - 148/00319 200×175 - 148/00320 |  |
| Mount as a pair with Telstra Radhaz sign no 2 | Telstra Radhaz sign # 8 (MERCS sign # 4) Material #: POLY 400×350 - 148/00323 200×175 - 148/00324 |  |
| Mount as a pair with Telstra Radhaz sign no 2 | Telstra Radhaz sign # 9 (No MERCS equivalent) Material #: POLY 400×350 - 148/00325 200×175 - 148/00326 |  |

NOTES:

1). 	RF EME warning Signs should not be installed on gates or external fences.

2). 	Note other than where mentioned all existing signage remain current until replacement is required" ie specifically, "DANGER RF Hazard Area" signs are "NOT VALID" and are "TO BE REPLACED".

3).	Telstra Sign No 8 & 9 are to be installed on High Powered sites.  eg. where Broadcast antennas are in-situ.

4).	Refer to MCF Document “7.1 Preferred Site Signage” on RFNSA “MERCS” document area for additional detail.

# EXTERNAL SIGNAGE: TELSTRA & 3GIS – Rooftops (EXCHANGES)

| Telstra and 3GIS site ROOFTOPS (EXCHANGES) |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
| Install this sign on the inside to access door to Telstra and 3GIS owned and controlled rooftops. May also be installed on a barrier where the roof top door is not the point of access restriction The barrier then becomes the point of access restriction Minimizes EME controlled area | Telstra sign # 6 No MERCS equivalent Material #: PVC 400×350 - 141/00626 200×175 - 141/00627 METAL 400×350 - 148/00319 200×175 - 148/00320 |  |

| Telstra and 3GIS site ROOFTOPS (EXCHANGES) continued |  |  |
| --- | --- | --- |
| Description | Sign Number | Sign |
| Install Telstra Radhaz sign # 7 along with Telstra Radhaz Sign No 6 (above) at rooftop sites where a fence or barrier has been installed to restrict access to a defined EME Hazard area. Mount in pair with Telstra radhaz sign No 2 Applicable to Telstra and Non Telstra sites | Telstra Radhaz Sign #2 Mercs sign # 5 Select size as required Telstra Radhaz sign # 7 MERCS sign # 3 Material #: Adhesive PVC 400×350 - 141/00628 200×175 - 141/00629 METAL 400×350 - 148/00321 200×175 - 148/00322 Telstra Radhaz sign #6 Material #: PVC 400×350 - 141/00626 200×175 - 141/00627 METAL 400×350 - 148/00319 200×175 - 148/00320 |  |

NOTES:

1). 	RF EME warning Signs should not be installed on gates or external fences.

2). 	Note other than where mentioned all existing signage remain current until replacement is required" ie specifically, "DANGER RF Hazard Area" signs are "NOT VALID" and are "TO BE REPLACED".

3).	Refer to MCF Document “7.1 Preferred Site Signage” on RFNSA “MERCS” document area for additional detail.

# EXTERNAL SIGNAGE: TELSTRA & 3GIS – MICRO CELL SITES

| Telstra and 3GIS Micro cell Sites |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
| Install one of these signs below antennas on street & power pole type installations (please note the three distinct signs for typical installation types). Note: Consideration should be given to not unduly alarming the passer by and the sign should be installed minimum 3m above AGL if possible. | Telstra Sign # 5 (A, B & C) No MERCS equivalent Select the sign appropriate to the EME exclusion zone. Material #: Adhesive PVC 5A: 130×100 - 141/00623 5B: 130×100 - 141/00624 5C: 130×100 - 141/00625 METAL 5A: 200×175 - 148/00313 5A: 145×100 - 148/00314 5B: 200×175 - 148/00315 5B: 145×100 - 148/00316 5C: 200×175 - 148/00317 5C: 145×100 - 148/00318 | 5A 5B 5C |

Notes

1).	Refer to MCF Document “7.1 Prefered Site Signage” on RFNSA “MERCS” document area for additional detail.

# EXTERNAL SIGNAGE: CUSTOMER END SITES – Masts and Poles and Roofmounts

| Customer End Sites (Mast, Poles and Roofmounts) |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
| Install one of these signs below or on the structure access point at a height of 2m AGL. | Telstra Radhaz Sign # 5 (A, B & C) No MERCS equivalent Select the sign appropriate to the EME exclusion zone. Material #: Adhesive PVC 5A: 130×100 - 141/00623 5B: 130×100 - 141/00624 5C: 130×100 - 141/00625 METAL 5A: 200×175 - 148/00313 5A: 145×100 - 148/00314 5B: 200×175 - 148/00315 5B: 145×100 - 148/00316 5C: 200×175 - 148/00317 5C: 145×100 - 148/00318 | 5A 5B 5C |

# EXTERNAL SIGNAGE: CUSTOMER END SITES – Roof and Gutter and other mounts

| Customer End Sites – Roof, Gutter and other mounts |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
| Install one of these signs below or on the antenna or antenna mount. | Telstra Radhaz sign # 10 No mercs equivalent Select the sign appropriate to antenna installed. Use where EME assessment has shown possible direct contact to be avoided. Material #: Adhesive PVC 10: 80 x 40 - 141/00638 Jabec Ref - tba Alternate is sign no 2 alone either on or adjacent antenna mount |  |
|  | TELSTRA Radhaz Sign # 11 (Mercs sign No 6) Use where EME measurement has shown no EME hazard area exists outside the dish itself May be extended to specific link services that meet the above criteria Material #: PVC adhesive 200 x 110 - 141/00639 Jabec ref TERH6640-V 100 x 55 - 141/00640 Jabec ref TERH6641-V |  |

# EXTERNAL SIGNAGE: NON TELSTRA STRUCTURE SITES

## Water Towers, poles and others

| Water Towers, poles and others |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
| Industry Standard Telecommunication facility identification sign Mount on gate to site or Q strap to pole | Telstra Radhaz Sign # 13 MERCS SIGN # 2 Material #: Polyprop 250 x 180 – 148/00331 JABEC ref TERH2331-P Metal 250 x 180 – 148/00330 JABEC ref TEMCF2330-M |  |

## Rooftops and accessible structures

| Rooftops and accessible structures |  |  |
| --- | --- | --- |
| For Non Telstra & 3GIS owned & controlled sites the responsibility of installing RF hazard signs is on the LICENCE HOLDER and therefore Telstra has a responsibility to ensure that appropriate sign have been installed. On Non Telstra & 3GIS owned & controlled sites the adjacent sign would be the most appropriate one to use. Note: As a guide refer to previous pages (p 4) for placement of signs depending on structure type. | Telstra Radhaz Sign # 4 MERCS sign # 1 Material #: PVC 400×350 - 141/00621 200×175 - 141/00622 METAL 400×350 - 148/00311 200×175 - 148/00312 |  |

| Rooftops and accessible structures continued |  |  |
| --- | --- | --- |
| Install Telstra Radhaz sign #7 sign along with Telstra Radhaz Sign No 4 (above) at rooftop sites where a fence or barrier has been installed to restrict access to a defined EME Hazard area. Mount in pairs with sign Telstra Radhaz sign No 2 Applicable to Telstra and Non Telstra sites EME general public limit is beyond barrier. Telstra Radhaz sign #4 installed on rooftop door | Telstra RADHAZ Sign # 2 Mercs sign # 5 Telstra Radhaz Sign # 7 MERCS sign # 3 Material #: PVC 400×350 - 141/00628 200×175 - 141/00629 METAL 400×350 - 148/00321 200×175 - 148/00322 Telstra Radhaz sign # 4 MERCS sign # 1 Material #: PVC 400×350 - 141/00621 200×175 - 141/00622 METAL 400×350 - 148/00311 200×175 - 148/00312 |  |

| RF sources – IBC’s and external antennas |  |  |
| --- | --- | --- |
| International Non Ionising Radiation sign. Used to indicate RF Hazard or source. | Telstra Radhaz sign # 2 MERCS sign # 5, 9 and 10 Metal 200x200x200 148/161 Jabec ref TERH2161-M PVC 100x100x100 141/185 Jabec ref TERH2185-V 75 x 75 x 75 141/ 413 Jabec ref TERH2413- V 40 x 40 x40 141/ 412 Jabec ref TERH2412- V |  |

## RF sources IBC’s and external antennas

## High Visual Amenity Sites

| High Visual Amenity sites |  |  |
| --- | --- | --- |
| Description | Sign No | Sign |
| Industry standard telecommunications site Identification sign Mount adjacent to concealed antenna sign or site access or hut door | Telstra Radhaz sign # 13 MERCS sign # 2 Material #: Polyprop 250 x 180 – 148/00331 JABEC ref TERH2331-P Metal 250 x 180 – 148/00330 JABEC ref TEMCF2330-M |  |
| Install this sign on shared facilities where the transmitting antennas are concealed from view. | Telstra Radhaz sign # 12 MERCS sign #7 Material #: PVC 250 x 180 – 148/00332 JABEC ref TERH7332-P Self adhesive 200×110 – 141/00641 JABEC ref TERH7642-V |  |

# Signage requirements of specific site owners:

Under some circumstances the landlord, facility manager or key stakeholder may require signage in addition or in place of that detailed in earlier sections of this document.  This section lists the specific signage requirements for the following parties.

- Victorian Electricity Supply Industry (VESI)- Microcells on electricity distribution poles

Defined under section 7 appendix C of MCF MERCS document “7.1 Preferred Site Signage” (available on the RFNSA under the “MERCS documents” folder).

Refer to this document for EME signage agreed between VESI and MCF as minimum required for all sites involving VESI infrastructure